Provor : a range of profiling floats for Operational Oceanography

G. Loaec - S. Le Reste - F. Marchese - A. Billant (IFREMER)

E. Le Marchand - T. Swinamer (MARTEC - METOCEAN)

Provor : the Marvor heritage

Based on experience

gained in developing & operating

multi-cycles MARVOR floats

 A 7 year at sea experience

 ~200 MARVOR floats deployed

 reliable, long life

Currents at a depth of 800 m in the Antarctic Intermediate Water mass.

Nearly 100 Marvor floats have been launched during the SAMBA experiment in the Brazil Basin between 1994 and 1997 for a 5 year mission

MARVOR floats statistics

143 MARVOR floats launched

for Arcane, Eurofloat, Samba experiments

MARVOR results: 65% still alive after 3 years

Provor - main features

- FSI or SeaBird CTD packages

- Self-ballasted, with active buoyancy control (hydraulic pump)

- Max profiling depth : 2000 m

- 100 to 150 cycles

- Possibility of data acquisition on down-profile to parking depth

- Great versatility in vertical sampling strategy (upper and deep layers with different resolution)

- ARGOS data transmission

Provor CT-F2

FSI Excell

Provor-CT-S

Seabird 41 CP

Provor T

Seascan

Provor range of floats floats

Metocean Provor CT-S

- SBE sensor

- ready to deploy (removable magnet)

- 150 cycles (36 lithium D-cells)

Sampling on down cast

Sampling on up cast

Select :

Upper and lower layers depth

Sampling interval in each

Parking depth

Timing of all actions

ARGOS parameters

Provor mission programme

Upper layer

Lower Layer

Parking Depth

Max. sampling depth  2000 m

Buoyancy control

- Transfer of oil between an internal reservoir and an external ballast

- No ballasting operation before launching

- +/- 30 dbars at depth

- About 2.3 liters of usable oil:

- emergence at the surface : >1 liter

- from surface to depth : 350 cm3 (function of difference of density)

- quality of machining : 500 cm3

Data Acquisition

- One sample every 10 seconds (1 sample per meter).

No power switching to avoid electrical noise

- From 2000 points to 100 to reduce the time at the surface

(using Argos data transmission system)

- 2 data reduction methods : chords or slices

Data reduction : chord method

. Segments between real points

. Maximum allowed error or number of points

. More points when variations are higher

69001 - Cycle 80

Data reduction : Slices Method

- 2 layers (upper and lower)

- each layer is divided into slices which height is user-defined

- all measurements which are gathered inside every slice are averaged (1 sample /meter)

Provor deployments

- On going experiments (North Atlantic)

- 1999-2000: technology experiment Pommier 	(5 profilers)

- 2000-2001: scientific experiment Pomme 		(16 profilers including 6 Provor CT-F2)

- GyroScope (funded by EU)   (40 profilers - PROVOR-CT-F2)

- deployments in 2001 and 2002

- Coriolis  (50 profilers - PROVOR-CT-F2)

- deployments in 2002

- Jamstec floats : 70 METOCEAN PROVOR-CT-S

- deployments in 2001 and 2002

Provor float deployment

from a ship of the French

Navy near the Azores