Iridium SVP-BTC80 transmission format Version 4 + BTC80

Marlin SVP-BTC80 Iridium buoys (2008)

| # Parameter | Bits | Pos | Min | Max | Formula |
| --- | --- | --- | --- | --- | --- |
| Mode | 3 | 0 | 0 | 7 | Fixed = 3 (011) |
| Observation time since 1st of January at 00:00 | 16 | 3 | 0 | 16382 | Hour (UTC) = n * 0.25 |
| Air pressure | 11 | 19 | 850 | 1054.6 | AP (hPa) = n*0.1 + 850 |
| SST | 9 | 30 | -5 | 35.8 | SST (°C) = n*0.08 - 5 |
| Pressure tendancy | 9 | 39 | -25.5 | 25.5 | dP (hPa) = n*0.1 - 25.5 |
| Submergence count | 6 | 48 | 0 | 100 | Subm. (%) = n * 1.6129 |
| Battery voltage | 6 | 54 | 5 | 17.4 | Vbat (V) = n*0.2 + 5 |
| Iridium transmission duration | 8 | 60 | 0 | 254 | SBDT (s) = n |
| 2nd Iridium Tech. parameter | 8 | 68 | 0 | 254 | ###### See table 2 |
| GPS fix time since 1st of January at 00:00 | 16 | 76 | 0 | 16382 | Hour (UTC) = n * 0.25 |
| GPS Latitude | 20 | 92 | -90 | 98 | Lat (deg) = n*0.00018 - 90 |
| GPS Longitude | 21 | 112 | -180 | 197 | Lon (deg) = n*0.00018 - 180 |
| 1st GPS Technical parameter | 7 | 133 | 0 | 126 | ###### See table 2 |
| 2nd GPS Technical parameter | 4 | 140 | 0 | 14 | See table 2 |
| Number of temp. probes | 5 | 144 | 0 | 30 | Num = n |
| Depth of temp. probe T0 | 8 | 149 | 0 | 0 | D0 = 0 |
| Temp. probe T0 | 10 | 157 | -5 | 35.88 | T0 (°C) = n*0.04 - 5 |
| Depth of temp. probe T1 | 8 | 167 | 0 | 254 | D1 (m) = n |
| Temp. probe T1 | 10 | 175 | -5 | 35.88 | T1 (°C) = n*0.04 - 5 |
| Depth of temp. probe T2 | 8 | 185 | 0 | 254 | D2 (m) = n |
| Temp. probe T2 | 10 | 193 | -5 | 35.88 | T2 (°C) = n*0.04 - 5 |
| Depth of temp. probe T3 | 8 | 203 | 0 | 254 | D3 (m) = n |
| Temp. probe T3 | 10 | 211 | -5 | 35.88 | T3 (°C) = n*0.04 - 5 |
| Depth of temp. probe T4 | 8 | 221 | 0 | 254 | D4 (m) = n |
| Temp. probe T4 | 10 | 229 | -5 | 35.88 | T4 (°C) = n*0.04 - 5 |
| Depth of temp. probe T5 | 8 | 239 | 0 | 254 | D5 (m) = n |
| Temp. probe T5 | 10 | 247 | -5 | 35.88 | T5 (°C) = n*0.04 - 5 |
| Depth of temp. probe T6 | 8 | 257 | 0 | 254 | D6 (m) = n |
| Temp. probe T6 | 10 | 265 | -5 | 35.88 | T6 (°C) = n*0.04 - 5 |
| Depth of temp. probe T7 | 8 | 275 | 0 | 254 | D7 (m) = n |
| Temp. probe T7 | 10 | 283 | -5 | 35.88 | T7 (°C) = n*0.04 - 5 |
| Depth of temp. probe T8 | 8 | 293 | 0 | 254 | D8 (m) = n |
| Temp. probe T8 | 10 | 301 | -5 | 35.88 | T8 (°C) = n*0.04 - 5 |
| Depth of temp. probe T9 | 8 | 311 | 0 | 254 | D9 (m) = n |
| Temp. probe T9 | 10 | 319 | -5 | 35.88 | T9 (°C) = n*0.04 - 5 |
| Depth of temp. probe T10 | 8 | 329 | 0 | 254 | D10 (m) = n |
| Temp. probe T10 | 10 | 337 | -5 | 35.88 | T10 (°C) = n*0.04 - 5 |
| Depth of temp. probe T11 | 8 | 347 | 0 | 254 | D11 (m) = n |
| Temp. probe T11 | 10 | 355 | -5 | 35.88 | T11 (°C) = n*0.04 - 5 |
| Depth of temp. probe T12 | 8 | 365 | 0 | 254 | D12 (m) = n |
| Temp. probe T12 | 10 | 373 | -5 | 35.88 | T12 (°C) = n*0.04 - 5 |
| Depth of temp. probe T13 | 8 | 383 | 0 | 254 | D13 (m) = n |
| Temp. probe T13 | 10 | 391 | -5 | 35.88 | T13 (°C) = n*0.04 - 5 |
| Depth of temp. probe T14 | 8 | 401 | 0 | 254 | D14 (m) = n |
| Temp. probe T14 | 10 | 409 | -5 | 35.88 | T14 (°C) = n*0.04 - 5 |
| Depth of temp. probe T15 | 8 | 419 | 0 | 254 | D15 (m) = n |
| Temp. probe T15 | 10 | 427 | -5 | 35.88 | T15 (°C) = n*0.04 - 5 |
| Depth of temp. probe T16 | 8 | 437 | 0 | 254 | D16 (m) = n |
| Temp. probe T16 | 10 | 445 | -5 | 35.88 | T16 (°C) = n*0.04 - 5 |
| Depth of end | 8 | 455 | 0 | 254 | Depth (m) = n |
| Unused | 1 | 463 | 0 | 0 | Always zero |

Table 1 - Dataformat

Columns :

- Mode is used to describe the type of data. It serves to determine which format must be used for GTS data. Meteo-France convention: “3” is used for buoy data; “1” for ship data.

- Column “Bits” : length of the word in bits for the parameter;

- Column “Pos” : Start bit in the message for the parameter;

- For some parameters, “Max” is just an approximation;

- Values for missing sensor or computation should be “all bits fitted to 1”;

- Total length of the total version 4 + BTC80 message : 464 bits i.e. 58 bytes, which includes 144 bits (version 4 data) plus 320 bits (BTC80 data);

|  | 2nd Iridium parameter | 1st GPS parameter | 2nd GPS parameter |
| --- | --- | --- | --- |
| ##### Marlin | Iridium transm. retry | Time to first GPS fix | Nb of GPS satellites |

Table 2 – Manufacturer’s choices for technological parameters

TTFF(s) = n * 2